HWB CLEANING SERVICES LLC

Construction Site Safety & Silica Dust Plan (CSSP)

Rough, Final & Touch-Up Post-Construction Cleaning Safety Program

| Document ID | HWB-EHS-002 |
| --- | --- |
| Version & Status | Version 1.0.0 ¦ ● APPROVED |
| Approval Authority | Humberto Dominguez, CEO |
| Date of Issuance | September 21, 2026 |
| Regulatory & Standard Alignment | CSI MasterFormat Div 01 35 23 & 01 74 00 ¦ OSHA 29 CFR 1926 |

1.0 Purpose & Construction Scope

This plan establishes mandatory safety protocols for HWB Cleaning Services LLC personnel performing rough, final, and touch-up post-construction cleaning on commercial construction sites. This document complies with General Contractor site safety requirements, owner specifications, and CSI MasterFormat Division 01 35 23 (Project Safety Requirements).

2.0 Respirable Crystalline Silica Dust Control (OSHA 29 CFR 1926.1153)

Post-construction cleaning involves drywall compound, concrete residue, tile dust, and mortar debris containing respirable crystalline silica. Inhaling silica dust causes severe irreversible lung illness (silicosis). HWB enforces strict engineering controls:

Zero Dry Sweeping Mandate: Dry sweeping with brooms or using compressed air to clean drywall or concrete dust is strictly prohibited under any circumstances.

HEPA Vacuum Extraction: All surface dust collection must be performed using commercial vacuums equipped with verified HEPA filtration certified to 99.97% efficiency at 0.3 microns, with sealed filter housings.

Wet Cleaning Methods: Baseboards, door frames, and window sills must be cleaned using damp microfiber cloths or wet-wiping systems to suppress airborne particles.

Respiratory Protection: Technicians must wear NIOSH-approved N95 or half-face respirators whenever dust-generating tasks warrant protection.

3.0 Mobile Elevating Work Platforms (MEWPs) & Scissor Lifts

All overhead dusting, conduit wipe-down, and elevated window washing utilizing scissor lifts or boom lifts must strictly adhere to ANSI/SAIA A92.20 and A92.22 standards, and OSHA 29 CFR 1926.453:

Certified Operators Only: Only technicians with an active, documented MEWP Operator Certification card issued or vetted by HWB management are authorized to operate aerial lifts.

Daily Pre-Operation Inspection: Operators must complete a physical walk-around checklist inspecting tire integrity, hydraulic fluid leaks, emergency lowering controls, horn, guardrails, and battery charge levels.

100% Fall Arrest Tie-Off: Operators in boom lifts must wear a full-body harness with an energy-absorbing lanyard attached to the manufacturer anchor point at all times. On scissor lifts, guardrails and entry chains/gates must be locked closed before ascending.

Barricades & Ground Spotters: A safety perimeter must be established using cones or caution tape below elevated work areas, and a designated spotter must monitor ground pedestrian traffic.

4.0 Glass Scraping, Window Cleaning & Blade Safety

Cut-Resistant Gloves: Technicians must wear cut-resistant gloves rated at minimum ANSI Cut Level A4 on the non-dominant stabilizing hand during all glass scraping activities.

Retractable Safety Scrapers: Only scrapers equipped with retractable blade guards are permitted. Blades must be retracted whenever the tool is not in active contact with glass.

Puncture-Resistant Sharps Disposal: Used razor blades must never be placed loose into trash bags. Blades must be placed directly into a hard-plastic sharps disposal container.

Glass Surface Lubrication: Glass surfaces must be thoroughly wetted with soapy water solution before scraping, holding the blade at a 30-degree angle to prevent scratches on tempered glass.

5.0 Industrial Ladder Safety (OSHA 29 CFR 1926.1053)

Duty Rating: Only heavy-duty fiberglass ladders rated Type IA (300 lb capacity) or Type IAA (375 lb capacity) are permitted on construction sites. Aluminum household ladders are forbidden.

Three-Point Contact: Technicians must maintain three points of contact at all times while climbing or descending ladders.

Top Step Prohibition: Never stand, sit, or climb on the top step or bucket shelf of a stepladder.

Inspection & Tagging: Damaged ladders must be immediately tagged 'OUT OF SERVICE' and removed from the job site.

6.0 Mandatory Construction PPE Standards

Hard Hats: ANSI Z89.1 (Type 1, Class E or G) worn continuously within active construction zones.

Eye Protection: ANSI Z87.1+ high-impact safety glasses with side shields worn at all times.

Safety Footwear: ASTM F2413 steel or composite toe work boots with puncture-resistant soles.

High-Visibility Vests: ANSI/ISEA 107 Class 2 or Class 3 fluorescent vests with reflective striping.

Hand Protection: ANSI Cut Level A2 to A4 work gloves.

7.0 Daily Safety Briefings & General Contractor Integration

The HWB Site Supervisor conducts a daily morning Job Hazard Analysis (JHA) covering active hazards (overhead trades, floor openings, scissor lift paths). The crew attends a 5-minute Tool Box Talk and signs the daily attendance log. The supervisor coordinates all work scopes directly with the General Contractor Superintendent.